# WRAPS EQUIPMENT/SOFTWARE REQUIREMENTS

# HARDWARE/SOFTWARE

## Servers

WRAPS requires four servers:  one Domain Controller, one Domain Controller/Exchange, one Application Server, and one Database Server.

Server configuration availability changes as the manufacturer upgrades equipment.  If the listed versions are not available, the current comparable versions should be substituted.

Following are the minimum configurations for each server:

## Server Hardware

### Domain Controller (e.g. HP Proliant DL320)

1 Dual Core Processor

4 GB RAM

2 – 80 GB SATA Disk Drives

CD-RW/DVD-ROM

### Domain Controller/Exchange & Application Server (e.g. HP Proliant DL370)

2 Dual Core Processors

4 GB RAM

6 – 146.8 GB 10K RPM Disk Drives

CD-RW/DVD-ROM

RAID Controller

Tape Drive for backups

Redundant Power Supply

Redundant Fan Kit

### Database Server (e.g. HP Proliant DL570)

4 Dual Core Processors

4 GB RAM

10 – 146.8 GB 10K RPM Disk Drives

CD-RW/DVD-ROM

RAID Controller

Tape Drive for backups

Redundant Power Supply

Redundant Fan Kit

## Server Software

### All Servers

Windows Server 2003 R2 Standard Edition

Anti-Virus Software

Microsoft .Net 3.5 SP1

Remotely Anywhere 9.0

Adobe Acrobat Reader

WinRAR

WinZip 10

Backup Software

### Application/Exchange Server

Microsoft Exchange 2003 SP2

Crystal Reports 9 Advanced SP4

PKZip Command Line

MSXML 6.0

### Database Server

Microsoft SQL Server 2005 SP3 Cu6

MSXML 6.0

## Server Rack

Standard 22U Rack

R5500 UPS

4 Port KVM Switch

Rack Mount Keyboard/Monitor/Mouse

## Network Components

Any standards compliant components will work, but the Refugee Processing Center supports Cisco as a networking platform.  WRAPS connectivity requires the following network components:

### Router

Cisco 1841 with rack mount kit and 3DES/AES encryption license, or

Cisco 2801 with 3DES/AES encryption license

### Firewall

Cisco ASA 5505 for sites with 10 or less concurrent users

Cisco ASA 5510

Note: Most RSC sites utilize the SonicWall PRO 2040 which is minimally supported by the RPC.  Cisco ASA is recommended.

### Switch

Cisco Catalyst 3560 10/100/1000 Ethernet Switch

Note:  Number of switches and port density are locally determined.

All networking components listed above have a single power supply, so an adequate battery backup is recommended.

## Required Peripherals (for WRAPS operation)

Network Printer(s)

Network Label Printer (Zebra LP2844 is utilized at the RPC)

Digital Camera

Double-sided Image Scanner

## Workstation Desktops

Each caseworker and each supervisor at the RSC must have at least a Pentium-class desktop computer with the following minimum requirements:

### Workstation Hardware

4 GB RAM

80 GB Hard Disk

CD/DVD-ROM Drive

4 USB Ports

17-in Monitor

### Workstation Software

Microsoft Windows XP Professional SP2

Microsoft Office 2007

Microsoft Internet Explorer 7.0

Adobe Acrobat Reader

Anti-Virus Software

## Structured Cabling

Cat 5E/6 UTP (plenum or low smoke)

Cable run from wall jacks to patch panel in wiring closet

RJ45 patch cables to connect PC to wall jacks

RJ45 patch cables to connect patch panel to LAN switches

## Security

Servers should be installed in a secure area to prevent unauthorized access to the server consoles

Router, firewall, and switches should be installed in a secure area to prevent unauthorized access to the management consoles

## Power

Two 15 Amp circuits

## Telecommunications

The RSC must have an Internet Service Provider (ISP) with a minimum of:

1024 Kbps dedicated link

IP subnet with at least 14 host addresses

## Personnel

RSC must supply personnel with the following minimum qualifications:

### Network Administrator

Microsoft Windows Server 2003 R2

Microsoft Windows XP Professional SP2

IIS 6.0

Microsoft SQL Server 2005 SP2

Microsoft Exchange 2003

Cisco Router/Switch/Firewall configuration and management

Microsoft Office 2007

### Caseworkers

Must be proficient in using e-mail, PCs, and Microsoft Windows applications

## Circuit Rides

The following equipment is needed to support the Circuit Ride functionality of WRAPS.  One set of equipment is needed to support a single circuit ride.

### Hardware – minimum of 2 laptops per Circuit Ride

Intel Pentium class processor (e.g. HP Compaq 6730b)

4 GB RAM

500 GB Hard drive – must be at least 3 times the size of the database

4 USB Ports

Workstations and switches for circuit ride processing are determined by local circuit ride requirements.

Wi-Fi is an acceptable networking solution provided that a minimum of Wi-Fi Protected Access (WPA) security is enabled.

Not required, but recommended for system backups is a 500 GB USB External Disk Drive – only 1 needed per circuit ride

### Software – one copy per laptop

Microsoft Windows Server 2003 R2 Standard Edition

Microsoft Office 2007

Microsoft SQL Server 2005 SP3 Cu6

Crystal Reports 9 Advanced SP4

Anti-Virus Software

PKZip Command Line

Microsoft .Net 3.5 SP1

MSXML 6.0

Remotely Anywhere 9.0

Adobe Acrobat Reader

WinRAR

WinZip 10

Backup Software

## Additional Information:

The RPC utilizes HP equipment for servers, desktops, and network printers.  Intel Xeon-based processors are preferred for server systems.  Both AMD and Intel-based processors are acceptable for desktops.  Also, Cisco is the preferred manufacturer for the networking/security infrastructure.  Any subscription-based anti-malware/anti-virus products should be sufficient.  Symantec is utilized at the RPC and has been verified to not interfere with WRAPS operations.